# SIT123: Data Capture Technologies

# Sensing Temperature and Humidity Activity Sheet

## Hardware Required

- Arduino Board

- USB cable

- DHT22 Temperature and Humidity Sensor (https://tronixlabs.com.au/sensors/humidity/dht22-temperature-and-humidity-sensor-australia/ )

- Male to Female Dupont Jumper Wires

## Pre-requisites: You must do the following before this task

Read this sheet from top to bottom

## Steps

- Connect your DHT22 Temperature and Humidity Sensor to the Arduino board. Refer to the images included below for guidance.

  - Pick a red male-female jumper wire and attach the female end to pin 1 (VCC pin) on the sensor. Plug the male end into the Arduino board’s 5V power pin.

  - Pick a blue male-female jumper wire and attach the female end to pin 2 (DATA pin) on the sensor.  Plug the male end into the Arduino board’s digital data pin 2.

  - Pick a black male-female jumper wire and attach the female end to pin 4 (GND) on the sensor. Plug the male end into the Arduino board’s GND pin.

  - Sensor’s pin 3 is not used.

- Connect your Arduino board to your computer using the USB cable.

- Open your Arduino IDE

- Go to https://github.com/adafruit/DHT-sensor-library and download the library to your computer by clicking ‘Download ZIP’.

- The .zip file you just downloaded is an external library. These have implemented functionality for DHT series of low cost temperature/humidity sensors. Using external libraries can save us from ‘reinventing the wheel’!.

- Save the downloaded library. You should save all your Arduino libraries in one location, such as inside Documents/Ardiuno/Libraries

- Go to https://github.com/adafruit/Adafruit_Sensor and download the library to your computer by clicking ‘Download ZIP’. Save it inside Documents/Ardiuno/Libraries

- Download the provided code at https://github.com/FeifeiDeakin/DHT22tempHum. If you download it in .zip format, you must extract it to a location on your computer after you download it.

See: https://support.microsoft.com/en-us/help/14200/windows-compress-uncompress-zip-files for extracting zip files on Windows &

https://support.apple.com/kb/PH25411?locale=en_US for extracting zip files on Macs.

- Go to the Arduino IDE. Select “File -> Open” and it will open a dialog box.

- Select the DHT22tempHum.ino file inside the DHT22tempHum folder & click Open.

- Add the libraries you downloaded to Arduino by selecting Sketch->Include library>Add Zip library. Select the downloaded zip libraries.

- In your Arduino IDE, click on the ‘Verify’ button. This will check for errors and compile your code.

- Use the “tools” menu to check the port to which the Arduino board is connected. Once the right port is selected, it is possible to upload sketches to Arduino.

- Now click the upload icon to upload the code to the Arduino board. If you get an error, check to be sure you’ve selected the correct device and port.

- Open the Serial Monitor in the Arduino IDE by selecting Tools->Serial Monitor, or by clicking on the Serial Monitor icon.

- Observe what’s printed on the Serial Monitor!

## References

https://github.com/adafruit/RTClib

https://learn.adafruit.com/adafruit-data-logger-shield/using-the-real-time-clock-3

http://www.instructables.com/id/How-to-use-DHT-22-sensor-Arduino-Tutorial/

https://plot.ly/arduino/dht22-temperature-tutorial/

http://cactus.io/hookups/sensors/temperature-humidity/dht22/hookup-arduino-to-dht22-temp-humidity-sensor